Loading Firmware

Select the unit to which you will upload the firmware:

- Remove J3 from Flashrom and turn on unit. Wait for the 4 lights under forwarding rate to remain on.

- Open the Config program and go to “FILE” and “OPEN CONFIG FILE”.

- Select the bin file you want to reload onto the unit.  You’ll need to go to “File of Type” and change from cnf to bin file to see the proper file to load.

- When prompted for a Serial Key, enter the Serial Key given to you by Karlnet.

- Go to File and select Upload Software.

- Hit Scan and change the IP address of the unit to something compatible to the IP address.  Note:  your IP address is found under Control Panel, Network in Windows 95/98.  Under XP run cmd.

- Continue through the 'Upload Software' screens.

- The unit will then reboot and reload the Firmware. You will be able to see a pattern of LED's. When the LED's have stopped flashing, the power light is green, and the interface lights begin flashing green again, the unit has begun normal Bridging operation.

- If you did not yet setup the unit with the required parameters, you should do so now.