# ROUTING EXAMPLE

# Example 1.  All IP Routers

One Base Station  IP address 198.17.74.1

Direct Route Ethernet 198.17.74.1

Radio 10.10.1.1

Three Satellites

- IP address 207.74.73.1

- IP address 176.45.34.1

- IP address 164.56.23.1

Each of Satellites would have routes:

Satellite A.  	On ethernet interface direct route of 207.74.73.1

On wireless interface direct route of 10.10.1.2

Default Route to 10.10.1.1

Satellite B.  	On ethernet interface direct route of 176.45.34.1

On wireless interface direct route of 10.10.1.3

# Default Route to 10.10.1.1

Satellite C.  	On ethernet interface direct route of 164.56.23.1

On wireless interface direct route of 10.10.1.4

# Default Route to 10.10.1.1

Note:  By setting the base station router as the default router, you wouldn’t have to set up indirect routes for the satellites.

The Base Station would have a minimum of these routes:

On ethernet interface Direct route of 198.17.74.1

On wireless side Direct route of 10.10.1.1

On wireless side Indirect route of 10.10.1.2, Target Router 207.74.73.1

Indirect route of 10.10.1.3, Target Router 176.45.34.1

Indirect route of 10.10.1.4, Target Router 164.56.23.1

The default router for the Base Station would be a router either in the same building or at the ISP for the network.

Preferred IP address would be the address of the ethernet card in each base and satellite.

# IP Masks could all be 255.255.255.0

Note:  Default Router would be the IP address of next router towards the Internet.