Wake on LAN

Control Panel > open Network and Sharing Center > choose “Change adapter settings” > Highlight Local Area Connection…then right click….choose Properties

left click Configure tab…..choose Power Management tab.

Uncheck  “Allow the computer to turn off this device to save power”

Click OK

Windows Firewall

Control Panel > open Windows Firewall

For  Home or work (private) networks  turn Firewall “OFF”

(you should already have a Firewall active in your home router)

Power Options

Control Panel > Power Options > make sure “Balanced” is chosen.  Click “Change plan settings”

Turn off display > Never

Put the computer to sleep > Never

Click  “Change advanced power settings”

Hard Disk >

Change “Turn off hard disk after” setting to “Never”

Sleep >

Sleep after “Never”

Allow Hybrid sleep “Off”

Hibernate after “Never”

Allow wake timers “Disable”

USB settings >

USB selective suspend setting “Disabled”

When finished with Power Options, be sure to click “APPLY”  (otherwise settings will resort back to defaults)

Then close Advanced Settings window

After making all changes then closing the Control Panel window,  be sure to completely power cycle the PC (completely shut down)…. not just Restarted…. before re-booting.