Mod procedure – see picture and text below.

Point 1 – TX Audio

Point 2 - Gnd

680 Ohm

2-Conductor 1/8” Phone Plug

180 Ohm

Mod procedure.

- Needed Items – 680 ohm 1/4w resistor, 180 ohm 1/4w resistor, 1/8” 2-conductor phone plug, 2” piece of small wire, small heat shrink.

- Open up supplied Yamaha Headset Battery box by removing 2 phillips scres inside – where batteries go.  Remove rear cover.

- Unsolder Red wire from Point 1 below.

- Solder 680 ohm and 180 ohm resistors in series – strip and solder one end of 2” piece of wire to junction of resistors.

- Solder unattached end of 680 ohm resistor to Point 1 below – where Red wire was removed.

- Solder unattached end of 180 ohm resistor to Point 2 below – this is ground.

- Strip unattached end of 2” piece of wire and put 3/8” piece of small heat shrink over wire.  Solder Red wire removed in Step 3 above to end of 2” piece of wire.  Shrink heat shrink over solder joint.

- Reassemble Yamaha Headset Battery box.

- Cut 1/8” 3-conductor plug off end of +/- 6” wire on Yamaha Battery box.  Strip off insulation and Snip Black wire – not used.  Strip ends of White and Red wires and tin with solder.  Slip cover for new 1/8” 2-conductor plug over wire, then solder plug to White & Red wires.  Red goes to Center / Tip of plug – White goes to Ring / Ground of plug.

- Enjoy…